COMMONLY USED TERMS IN CCTV OPERATION

- AGC -Automatic Gain Control-an amplifier inside the camera it switches in automatically if the output signal goes low.

- Analogue -an electronic signal of varying voltage that corresponds to something in the real world.

- Auto iris - an automatically controlled camera lens (more commonly DC driven), that adjusts to a varying Iris size dependent on the amount of light that it detects.

- DC Driven Auto Iris – More commonly used today to interface with cameras that have extra programming features.

- CCD -The Digital chip in a digital camera.  This chip this more commonly a 1/4 or 1/3 CCD chip.

- Duplex -the ability to perform to functions simultaneously.  Usually this means being able to watch live video at the same time as watching recorded video.

- Triplex -as above but adding the ability to record August watching live and recorded video.

- Composite -the combination of video signal, blanking signal and synch signal all sent down one cable. This is the opposite of Y/C cable.

- Field -1/2 of a complete picture (or frame) containing all the odd or even lines.  There are 50 fields per second.

- F-stop -a number that describes how open the Iris of a leans this.  Each increment of F-stop effectively halves the amount of light getting into the camera.

- Frame -one complete picture consisting of 2 fields. There are 25 frames per second.

- Lux -the measure of light, or amount of light.  1 Lux = 1 candela.  This equates approximately to the light of one candle.  Whilst a professional photographer would measure the light source at the subject, CCD cameras are more likely to measure the light at the camera lens.

- Gain -a measure of amplification expressed in decibels.

- Gamma correction -This allows the change in the linearity perceived by human vision.  It helps to increase contrast of dark areas if gamma = 0.45.

- Ground loop-Hum bars on a monitor that are created when two separately grounded points in an electrical system create an unintended current path.

- Genlock -an electronic circuit in a camera that enables it to be synchronized to an external source.  Helps remove monitor role when switching between cameras. often occurs where the monitor is some distance away from the recording unit and when both units are powered up the resistance of earthing differs at both sites.

- Impedance -The combined resistance and reactance in an electrical circuit expressed in Ohms.   CCTV systems operate on a 75 ohm impedance.

- I S D N -Integrated Services Digital Network.  Part of a digital telephone network that allows a dedicated telephone line between premises.

- Line lock-where the video signal synch pulses are referred to the camera supply frequency.

- Lux-the measurement of light emitted by one candle in one square meter.

- Microwave-short frequency signals that travel through the air-used to transmit video and data signals.

- Multiplex-the technique of recording several video cameras sequentially at speed on to a single video tape.

- Phase adjustable line lock-ability to move the synch reference of a camera locked to an AC supply frequency.  Used to stop picture role.

- Pixel-the smallest individual dot that can be separately illuminated on a display screen.

- Resolution-ability to define alternate black and white lines on 2/3 the area of a monitor screen.

- Sensitivity-the ability of a camera to operate in low light.

- Sensor-C C D chip capable of changing light into electronic signals.

- S/N ratio-the amount of noise compared to the amount of video signal.

- S-V H S-high resolution vision of the standard V H S systems.  Requires a component [Y/C] input into therefore requires two cables from camera to monitor.

- Synch-the signals it used to buy a video system for visual information to be displayed coherently.

- Varifocal: (zoom) lenses solve the problem of choosing the correct focal length for the job.  Varifocal lenses can be manually adjusted (zoomed) to the required angle of view when setting up the camera.  Available in both Manual and Auto Iris.

- White balance-Term used to explain the way all colors are referenced to white.  White appears to have a different tone under differing light conditions e.g streetlights.