# Frequently asked questions

Digital Sprite has individually programmable activity grids for each camera

Each activity grid has a 128 (16 x 8) cell matrix with five sensitivity levels. Activity detection can be configured to record cameras more often, display active cameras in full screen, close a relay and log the activity as an event.

Up to 8 alarm inputs per channel are available via remote alarm modules.  They can be configured to record cameras more often, display alarms in full screen, close a relay, log the alarm as an event, and send PTZ cameras to a pre-set position.

A settable day, night or weekend schedule can automatically:

* Remove or add cameras into the record sequence. e.g. record only outdoor cameras during the night.        Switch alarms or activity on and off.
* Select whether alarms and activity are interleaved, recorded exclusively or recorded as normal.
* Alter the record rate for alarms and activity detection – increasing the number of pictures recorded each second when an event is triggered.

Multitasking Operation
Recording is never interrupted, no matter what feature is being used.

Fast image retrieval
Images are stored onto internal hard disks for near instant image retrieval, removing the need to forward and rewind tapes.

Event log with 'Active Thumbnail' view All internally stored events are logged by time, date, event type and camera number. Selecting any of these events displays a preview window which plays back the whole event in a loop.

Adjustable image quality Image quality can be adjusted by the user to provide performance equivalent
to VHS, S-VHS and better than S-VHS.

Zero image degradation
Images stored on the hard disk can be replayed repeatedly without affecting the quality.

One box recording solution
Everything required to record and playback images from the disk is included in one unit. External tape and disk archiving is optional.

Telemetry
In-built coaxial telemetry controls both Pelco and Dennard domes directly, and most major manufacturers domes via a BBV RX-100 converter.

New Features
Digital Sprite multiplex recorders are now available with:

* Built-in JVC , Ultrak and Kalatel dome protocols.
* High capacity instant access RAID HDD storage.
* E-mail notification on alarm.
* Thumbnail review via the network.
* Protected event partition.
* View hidden cameras via the network.
* Up to 150GB internal HDD storage.
* Simultaneous record and playback.
* Easy-to-use CCTV interface.
* Live network video option.
* Up to 2,250GB instant access RAID storage.
* Integrated control of JVC, Ultrak and Kalatel domes.

Integrated Telemetry:

No extra conversion equipment or additional keyboards are now required to control the domes. Simply operate them via the Digital Sprite's keyboard.

Easy set-up
No need to install additional conversion equipment. The Digital Sprite connects straight to the dome.

Easy usage
Digital Sprite's keyboard now controls the dome's menu 

Network interface
Control of the dome's menu is also available direct from the network
interface.

Compatible domes

include the JVC TK-C675BE, JVC TK-C553E, VCL Orbiter Microsphere, VCL Jupiter Microsphere and the Ultrak UltraDome KD6.

Raid Storage:
Digital Sprite now gives you instant access to additional high capacity storage with the Zero-D RAID available from Dedicated Micros.  The Sprite Scuzzi Port is a Passive Terminator.

What's a RAID?
A RAID (or Redundant Array of Independent Disks) contains hard disks that add to the internal storage capacity of the Digital Sprite.

Increased storage capacity
Zero-D RAIDs give you up to an extra 5 weeks storage (at 6pps & 18KB file size). 

Instant access
The RAID is linked directly to the Digital Sprite, giving instant access - images can be retrieved and viewed as if they were in the Digital Sprite itself. 

Easy set-up and use
The RAID simply connects to the back of the SCSI port. Digital Sprite then sees all the storage capacity as one big disk space, and removes the hassle of tape changing. 

Ability to recover data
Zero-D RAIDs come pre-configured at RAID level 5, which gives you the extra assurance that if in the unlikely event of disk failure your data is copied onto the other RAID disks for instant retrieval. Disks can be hot swapped without effecting recording. 

Choice of storage options 
Digital Sprite works with an IDE-SCSI RAID. Zero-D RAIDs have 6 bays, come in a choice of tower or rackmount and have 4 different disk capacities ranging from 150GB to 375GB. 

Multiple RAID option 
A single Digital Sprite can have up to a maximum 7 external storage devices.  For example 6 RAIDs and 1 Zip / Jaz drive offers 2,250GB capacity, giving 30 weeks extra storage.