GENERAL DIGITAL CAMERA INFORMATION

- CCD cameras:	The letters CCD stand for charged coupled device.  This basically explains how light is grabbed by a pixel and then sent to a storage it is coupled to.  It is then seen from the storage area to be processed into an electronic image.  CCD’s are available in 1/8, ¼, 1/3, ½ formats

- Frame Transfer:  There are two ways of electronically processing an image using a C C D sensor.  The first is called frame transfer and these sensors have two distinct parts to them that are easily discernible to the naked eye, the image gathering area and the storage area.  They operate by fist grabbing the viewed image and then moving it vertically down the Sensor into the storage area.  This has the problem that if a bright point of light hits the sensor face then it travels down the screen as a " smear ".

- Interline Transfer:  Interline Transfer CCTV sensors and now the most popular form  of CCD device.  This form of chip this a much smaller than the frame transfer chip.

- Sensitivity:  This is the figure given by a manufacturer of a camera.  This can be measured at the camera face without a lens or with a lens.  Sometimes it is measured at the scene!  Obviously this affects the amount of light available to the camera chip.  As such the sensitivity levels claimed by camera manufacturers will vary depending on the measurement method used.

The correct method for specifying camera sensitivity should be to quote the camera with a lens of known light capability.

A specification should look something like this: 0.3 Lux at f1:4

As a general rule following would apply:

- direct sunlight			100,000 Lux

- full daylight	 		10, 000 Lux

- overcast day			1,000 Lux

- dull day/well lit office		500 Lux

- very dull day/floodlit carpark	100 Lux

- twilight/street lighting		10 Lux

- deep twilight			1 Lux

- full moon			0.1 Lux

- Signal to noise ratio:  This is expressed by decibels [dB] and is an expression of noise shown by the camera as compared to the amount of signal.  It is compatible to the amount of hiss between a good and a poor quality tape player.

- Synchronisation:  Synchronisation of cameras is important so that camera " roll " on monitors can be avoided.  There three separate ways of synchronizing cameras powered by an AC power supply.

-	Line lock.  This is where all cameras are powered by the same AC power supply. Line-lock is not possible on internally synchronized 24 VAC cameras.

-	Adjustable or phased adjustable line lock.  Ideally this requires the use of an  oscilloscope on setup.  1 camera is used as a reference camera and all other cameras have their 	reference point moved so they are in line with the reference camera.  The oscilloscope is placed at 	the recorder end in the reference camera provides one wave form and the camera being addressed 	provides the second wave form.

-	Gen lock or " Generator locked ":  this is the most definite method of locking cameras.  	A synch generator or a reference camera is used to give a definite frequency to all cameras in the 	system.  However a second cable is required to go to each individual camera to provide the synch 	reference.

-	Note:  DC cameras have no AC supply frequency to lock on to.  They rely on their own internal Synchronisation named "Crystal lock".   Avoiding monitor role is often difficult with DC cameras.

- Lens mounts:  "C" mount lenses are old technology.  They are still however very popular on ½ inch format lenses.  CS mount lenses are the standard today and use 1/3 inch format lenses.    Back focus can be used to to physically move to the lens away from the sensor.

- Power supplies:  common power supplies are 240  volt AC, 24 volt AC, 12 volt DC or co-axial powered.  Co-axial powered cameras do incurr problems when the system is required to be expanded.   It is difficult to include new concepts in to a line powered CCTV system as there is no  1 V Pk-Pk signal available.

- White balance:  The eye is excellent at processing an image.   Cameras find it more difficult.  Where as an eye can see that a white car under an Orange Street light is a white car.  Without white balance camera would be confused.  The lens white balance allows such an object to be shown on a monitor is it would be perceived by the human eye.

- Aperture:  A typical camera specification may read 1.5 Lux at F1: 4.  what the F1: 4 is  stating is that the Iris of the camera can open to a certain known extent  The equation is the F number equals the focal length of lens divided by the F number.  This equation states that when the F number is small, the aperture is large.  The F number allows half the amount of light to pass on to the camera sensor as the previous F number.  For example:  F1:2 allows twice as much light to pass through as F1:4.  therefore a camera specified as being 1.5 Lux sensitivity that at F1:2 is exactly the same as a camera quoting 3 lux at F1:4.  in fact as F1:4 is nearer to the industry standard then the second camera is better specified.

- Co axial cable:  The most common method of transmitting video signal.  The most common types used are RG 59 and RG 6.

- distance -RG 59 allows a video signal to travel 200-250 m.  RG 6 has better shielding and a larger core size and allows the signal to travel 400-450 metres w/o amplification.

- amplification-video amplifiers can be used to boost the video signal up to 950 metres 	on RG 59 and 1, 400 metres on RG 6.  Amplifiers can not be used with co axial telemetry.

- Twisted pair:  this form of signal transmission can be used when cable has to travel through noisy environments or over longer distances.  A standard video signal goes down one of the twisted pair cables and a reverse image of the video signal goes down the other cable.  They are added together at the receiving end in employees that only goes in one direction is cancelled.  Systems can be distances of 600 to 1,000 metres and repeaters can be used to obtain much greater distance.

- Fibre Optics:  A piece of stretched glass used for all types of data transmission as well as video transmission.  It operates alone the distances, there is no perceptible interference, it does not corrode and is very chemical resistant.  The disadvantages of fiber optic systems are that they are sometimes difficult to design and that special tools and skills are required for an installation.

- Microwave:  This type of free space transmission does not suffer from degradation due to weather.  Line of site is required but due to the width of the microwave there is an allowance for accuracies within a few meters.  Microwave links of 10 km or more are not uncommon.  It is also possible to send data, speech and controls for Pan and tilt cameras alongside video signals.