ENCRYPTION STATEMENT

Digital Signing of Dedicated Micros Digital Sprite Images

The Digital Sprite (DS) compresses digital video images using the JPEG algorithm. During this processing the DS adds markers and header constructs which allow non-video information concerning the video image to be stored along with the JPEG video data. Importantly, each JPEG image is in itself complete and does not require reference to other images in a sequence in order to be reproduced.

The header data that is appended to the image includes - amongst other things;

- The date and time the image was recorded.

- Details of the camera to which the image relates.

- Details concerning the DS unit on which the image was recorded (Unique MAC number).

- A unique, monotonic increasing (by one), sequence number for the image.

- Space for a 128bit digital signature.

From Software Version 2.0 (DS only) images are signed as they are written to the DS hard disk.

The nature of JPEG compression means that any change in the image data carried out on the compressed data will be detectable. It is not possible to alter any of the header details without affecting the signature.

The digital signing algorithm is a public domain, proven, algorithm that is sensitive to the slightest change in the source data. It will generate a vastly different result if only a single bit changes.

It is therefore possible to confirm the following when using the header data and digital signature contained in each image and by inspection of the image

- That the header data has not been altered

- Using the sequence number, that no image has been removed from or inserted into the sequence

- That the video data has not been substituted with that from another image.