### Paradox Spectra Operating and Use Instructions

(For more detailed information please refer to the booklet)

| Before leaving ensure all doors are locked, windows closed and pets outside if you don’t have pet sensors. A common reason for “false alarms” is hot air coming through a window and being picked up by the sensor. |  |
| --- | --- |
| Set alarm when leaving | Ensure all zone lights on keypad are out. Press and hold the FORCE key until the system beeps 3 times and the ARM1 light comes on. Leave the house. If you return for any reason unset the alarm and re-set when you leave. |
| Unset the alarm when you arrive home | Confirm that alarm is armed - the ARM1 light is on. Enter your 4 digit pin number. The system will beep 3 times and the green READY light will come on. |
| #### Stay arming (Protects one area of home while you stay in another.) | Press and hold the STAY button until the system beeps 3 times. The ARM1 light stays flashing. |
| Unset stay arming | Put in your 4 digit pin number. |
| Smoke Alarm goes off | Siren sounds in pulses. In case of a real fire evacuate all people and pets immediately. If alarm is accidentally activated enter 4 digit pin number. If siren sounds again ensure any smoke is cleared from the detector and enter pin number again. |
| Panic 1 & 3 | This is a panic alarm to ADT/Armourguard for a personal emergency |
| Panic 4 & 6 | This is a panic alarm to ADT/Armourguard for a medical emergency |
| Panic 7 & 9 | This is a panic alarm to ADT/Armourguard for a fire |
| System stops beeping on setting and unsetting alarm | The mute has been accidentally turned on. To fix the problem press and hold the CLEAR key until the systems beeps once. |
| System is beeping every 10 seconds | Open keypad door and you should see the TBL button lit up. Press the TBL button and note which number lights up. Look at the list of trouble conditions and action as necessary. To stop the beeping now press the CLEAR button. The TBL button will remain lit until the situation clears itself eg if a power failure, the power coming back on. |
| Keypad beeps when zones are activated | Determine which zones are beeping. Press and hold the number relating to that zone until the system beeps. |
| To change the MASTER CODE | This is your main 4 digit pin. To change : Press ENTER then YOUR 4 DIGIT PIN then 0 0 1 then enter your NEW 4 DIGIT PIN then press CLEAR. Test the new number. If correct it will set the system, then enter in again to unset. Take care as the first four buttons pressed become your new pin. |