SPECTRA USER HELP GUIDE

Q…Why does my keypad beep when I move around the house?

A…Chime zone is enabled,press corresponding key on keypad until you hear a long beep.

Q…Why do I only hear 5 seconds of my exit delay ?

A…Keypad is muted. Press and hold the CLEAR key for 3 seconds.

Q…MEM light is showing on my keypad.

A…There has been an activation,press the MEM key to show which zone has activated.Press CLEAR to exit.

Q…BYP light is showing on my keypad.

A…A zone is bypassed.Press the BYP key for 3 seconds to display the zone that is bypassed.

Q…TBL light is showing on my keypad.

A…Press the TBL key and tell me what you see.

- Battery is low ……….requires a service call

3  Mains failure………...Is power on in the rest of the house? If not,then wait until power is restored.

7  Communication failure…..requires a service call

10 Loss of telephone line….are you calling us on a cellphone ?

Q…I need to reset my smoke detectors,how do I do it ?

A…Press and hold the PG key for 3 seconds.

Q…How do I programme a user code into my system ?

A…Enter…(MASTER CODE) 3 digit (SECTION) enter a 4 or 6 digit code.