Common problems/calls relating to Spectra Panels

- Panel answers incoming calls on a regular basis.  Check that ring count (sect 078) is at 15 rings.  To disable panel answering after 15 rings, enter 000.  Also check answer machine override time.  If you don’t want the feature, leave section 077 at default (000).  If you do require, select 032. In busy shops or some households, it is not practical to use the answer machine override features due to lot of activity on the phone.  With both sections 077 and 078 set to 000, the lonely way for the panel to answer a call (including Winload) is to have the end user force answer the phone.  To do this, they press:  ENTER  then their MASTER CODE then FORCE.

- Panel grabs the line and cuts me off in the middle of a call.  Could be due to excessive loading on the phone line.  If the line voltage drops below 3.5V the panel will bring up a telephone line trouble.

- Zone 6 and 7 show as being in alarm when not being used.  Check that options 7 & 8 are not turned on in section 128.  This is keypad supervision.  To use this feature, you must still seal the keypad zone with a 1K resistor if J2 is removed, or with a small piece of wire if it is left on.  Remember to remove J1 on the second keypad.

- Keypad 2 zone will not clear.  Remember to power down the complete system when changing keypad jumpers (VERSION 1 ).  The panel looks at the state of the jumper at power up to recognise keypad 1 & 2.  Also make sure zone input is properly sealed, taking note of the J2 setting for EOL resistor.

- Zone indicators are on, but I am not using the zones. If you are not using a zone, make sure it is either sealed or delete the zone from programming (sect 001 – 016 for zone 1-16). To delete a zone, make the first digit for the zone type “FORCE”.

- Keypad beeper seems to die. Most likely the keypad has been muted.  In mute mode, beeper will sound for the first 6 seconds of exit delay, but make no noise on entry delay of activations. Press and hold the “CLEAR” key for around 3 seconds, and you should hear a long beep.

- Noise on phone line. As the incoming line is polarity sensitive, it has been found that earth problems can show up as a humming type noise on the phone line.  Often reversing the polarity of the wires in the TIP and RING inputs can solve this.Also try disconnecting the dialler earth as this can induce earth loop currents if the local exchange has a different earth reference point.

- Dialler is not working. There could be a multitude of reasons for this.  Things we have discovered are:

  - If either the phone number, account code or reporting is not entered correctly, the panel will not even grab the line

  - If wrong information was programmed in for either the phone number or CS number, events will be stored in the panels buffer with those values.  If you correct either of these, you will need to clear the dialler buffer by going into programming mode and pressing the STAY button.  This will cause the phone line to hang up, and clear all events waiting to be reported.

  - The phone line in and phone line out are round the wrong way.

  - Protection MOV’s are blown.  This is caused by excessive transients down the phone line.  In some cases, people have connected the 230vAC to a lighting circuit where the green wire is used as a return for 2 way switching.  Under some conditions, this can send 230V into the earth terminal!

  - If using a plug pack, make sure the wires are in the correct place.  We have seen a site that had accidentally inserted the green and blue wires into the AC input, and the red AC wire into the earth terminal.  Things initially worked OK, but the telecom line failed after several hours due to the earth reference sitting at 16v.

  - In some rural areas, Telecom have devices that can send up to 16 phone lines down one pair of conductors. To be able to do this, they modulate each line and send it down the copper to be de-modulated at the other end.  The time lag this creates makes it impossible for contact ID signals to work.  In these cases, you must report using 4+2 formats.

  - Make sure you are not on an ISDN or Jetstream line.  For these to work, you cannot grab the line as it comes off the street.  Instead, you have to connect at the house side of the interface box.

- House phone is dead. Most likely cause is a telecom fault. Telecom have the annoying habit of disconnecting the alarm system which seems to fix the fault.  They blame the alarm, yet in nearly all cases the alarm is not at fault.  To confirm if it is the panel that has grabbed the line, press the enter button followed by either the installer or master code.  Press the STAY button.  This will make the panel hang up if it is actually holding the line.  If the line is released, the cause might be that the panel has answered an incoming call, and for some reason been unable to hang up the line (could be noise on the line).  If this is the case, perhaps the ring count and answer machine override times need adjusting as per point 1 above.

Simple Telephone line Tester

Connecting the wires across the T & R on the panel will allow you to listen in to the communications.

The switch across the capacitor is an option allowing you to test for dial tone.  Pressing the button will cause enough load to give you dial tone.   If there is no dial tone, the line is bad!

When listening in on a transmission, you should hear the phone dialling, followed by the receiver answering it.

The receiver will give two tones, firstly a 1400hz followed by a 2300hz (referred to as the handshake).  At this point, you should hear the dialler sending a burst of 16 DTMF tones.  Immediately after the panel sends the tones, you should hear a 1400hz “kissoff”.  If the panel has additional data to send, the process will be repeated.

Once the panel has sent all the data it has, it will hang up after the last kissoff.